# Wind Time series for WASA2 domain, South Africa

## Metadata and further information

## July 2020

| METADATA |  |
| --- | --- |
| Data set name | Wind time series for WASA2 domain, South Africa |
| Data set date production | July 2020 |
| Data provider | DTU Wind Energy, UCT and CSIR |
| Contact persons | Chris Lennard (UCT) and Andrea N. Hahmann (DTU) |
| Contact details | lennard@csag.uct.ac.za (UCT) or ahah@dtu.dk (DTU) |
| Data | Time series of 2-meter temperature and wind speed, wind direction at 20, 60 and 100 meters. |
| Data format | ASCII |
| File name(s) | TS_<latitude>_<longitude>.csv |
| Data origin | WASA2 WRF simulation 3.33km x 3.33km model domain (see WASA2 Mesoscale Report on WASA download site for more details). |
| Time period covered | 07 January 2010 to 24 January 2018. |
| Spatial resolution | Every 30km. The WRF WASA2 model simulation outputs data every 3.3km in the final domain. Each time series coordinate is a sub-sample of these data spaced at every 30km. |
| Time resolution | Every 30 minutes. |
| File size | 16.3MB per file unzipped; 2789 files; 44.5 GB in total Zip file with all locations: 14GB (compressed) |

| DATA PARAMETERS |  |
| --- | --- |
| Latitude | Latitude in decimal degrees |
| Longitude | Longitude in decimal degrees |
| Wind speed | Wind speed [ms-1] |
| Wind direction | Wind direction to north [degrees] |
| Temperature | Degrees Kelvin |
| Date and time | YYYY-MM-DD_HH:MM:SS, where time is in UTC hours |

| FILE STRUCTURE |  |
| --- | --- |
| Column names | Times, T2, WSPD_0, WDRO_0, WSPD_1, WDRO_1, WSPD_2, WDRO_2 |
| Times | Times: YYYY-MM-DD_HH:MM:SS, where time is in UTC hours |
| T2 | Temperature at 2 meters (degrees Kelvin) |
| WSPD_0 | Wind speed at 20 meters AGL [ms-1] |
| WDRO_0 | Wind direction at 20 meters AGL [degrees] |
| WSPD_1 | Wind speed at 60 meters AGL [ms-1] |
| WDRO_1 | Wind direction at 60 meters AGL [degrees] |
| WSPD_2 | Wind speed at 100 meters AGL [ms-1] |
| WDRO_2 | Wind direction at 100 meters AGL [degrees] |

| COORDINATE SYSTEM |  |
| --- | --- |
| Projection | Lambert conformal projection |
| Centre of grid | -28.70254°N, 24.75159°E |
| Grid rotation | Standard longitude: 10°W |

| TECHNOLOGY |  |
| --- | --- |
| Mesoscale Modelling software | WRF Version 3.8.1 (see WASA2 Mesoscale Report on WASA download site for more details) |
| Data post-processing | DTU Wind Energy and UCT-CSAG software. |

DETAILED NOTES

Purpose This data set was created for the WASA project and the Department of Mineral Resources and Energy, South Africa. The wind time series were designed to give information on annual, seasonal and diurnal variations of wind speed and direction for across South Africa. Methodology Reference is made to the information and documentation available from www.wasa.csir.co.za. Limitations The operational envelopes of the wind atlas methodology and the WRF model limit the accuracy of the data set. The timeseries data were designed to be used for: - Study the annual, seasonal and diurnal variations in wind resources - As input to power system modelling - Study the geographical cross correlation of wind across South Africa - Used for long-term correction of the wind resources given by the WRF wind climate files (“lib” files) NB! These timeseries data are not intended for wind resource calculations except for the above. For wind resource calculations users are referred to the Verified Numerical Atlas and Guide on the download site and Metadata Wind Resource Map (WRF). Available documentation The Numerical Wind Atlas for South Africa is a product of the Wind Atlas for South Africa project and is described further on the WASA download pages.

DISCLAIMER

In no event will the WASA Partners or any person acting on behalf of the WASA Partners be liable for any damage, including any lost profits, lost savings, or other incidental or consequential damages arising out of the use or inability to use the information and data provided in this data set, even if the WASA Partners have been advised of the possibility of such damage, or for any claim by any other party. The principles, rules, exclusions and limitations provided in the Disclaimer on the WASA download site apply to the data set described here as well, even though this data set may not be distributed via the web site. By using this data set, you agree that the exclusions and limitations of liability set out in this disclaimer are reasonable. If you do not think they are reasonable, you must not use this data set.